Implementing and Analyzing GPIO Programming: Blinking the Onboard LED Using STM32 HAL

Step 1: Create a New Project for Nucleo-H753ZI

Go to filenew

New STM32 Project

Click on the STM32 Project to create a new project

File  New  STM32 Project

Now here the target selector will be opened

Go to click on Board Selector

Enter the board configuration for example now the present experiment is done with NUCLEO-H753ZI

Enter that in commercial Part Number

Now click on the NUCLEO board as shown and click Next

After clicking next we get a page  Enter the Project Name example here I kept Lab02_H753_LED_Blink.

Click on next and then Finish

Click yes for the all peripheral if it appears

The new project is created

Now unable the required pins

In the graphical Pinout View, look for pin PE1

Click on pin PE1 and select GPIO_Output.

How to enable? PE1GPIO – OUTPUT

Click on the PE1 in the board and select the GPIO- OUTPUT

Go to the System Core menu on the left pane and click on GPIO.

Click on PE1 in the pin list table at the bottom to view its configuration parameters.

After clicking on PE1 we get the PE1 Configuration

- GPIO output level: Low

- GPIO mode: Output Push Pull

- GPIO Pull-up/Pull-down: No pull-up and no pull-down

- Maximum output speed: Low (or Medium for faster transition speeds)

After that save(Ctrl + S) and generate code  click yes

Click yes

The code is generated will look like this

- Scroll down until you find the while (1) infinite loop inside the main() function.

- Add the toggle and delay lines inside the loop so it looks like this:

Enter the code in the while(1) loop

while (1)

{

/* USER CODE END WHILE */

/* USER CODE BEGIN 3 */

HAL_GPIO_TogglePin(GPIOE, GPIO_PIN_1);

HAL_Delay(500);

}

Click the Hammer (Build) icon at the top toolbar and make sure the console reports 0 Errors, 0 Warnings.

- Connect your Nucleo-H753ZI board to your laptop using the USB cable.

- Click the Run/Debug (Green Play bug icon) button to flash the code onto the board.