Experiment 5: Evaluating GPIO Drive Strengths: Performance Analysis of GPIO Speed and Pull Up/Pull-Down Configurations

Board Used: NUCLEO-H753ZI (STM32H753ZI)

Course Title: EMBEDDED SYSTEM DESIGN AND IOT

Semester: 2026-2027 ODD SEM

Course Code: 25EC2206E

Step 1: Create a New Project for Nucleo-H753ZI

Go to File → New → STM32 Project

Click on the STM32 Project to create a new project.

File → New → STM32 Project

Now here the target selector will be opened.

Go to Board Selector.

Enter the board configuration. The present experiment is done with NUCLEO-H753ZI.

Enter NUCLEO-H753ZI in the board search / selector.

Now click on the NUCLEO-H753ZI board and click Next.

After clicking Next, enter the Project Name. Example: Exp5_GPIO_Drive_H753ZI.

Click on Next and then Finish.

Click Yes for all peripherals if it appears.

Step 2: Configure PA6 as GPIO Output

The new project is created.

Now enable the required pin.

In the graphical Pinout View, look for pin PA6.

Click on PA6 and select GPIO_Output.

How to enable? PA6 → GPIO_Output.

Click on PA6 in the pinout and select GPIO_Output.

Step 3: Configure PA6 GPIO Settings

Go to the System Core menu on the left pane and click on GPIO.

Click on PA6 in the pin list table at the bottom to view its configuration parameters.

After clicking on PA6 we get the PA6 Configuration.

Set the following configuration for the first test:

GPIO output level: Low

GPIO mode: Output Push Pull

GPIO Pull-up/Pull-down: No pull-up and no pull-down

Maximum output speed: Low

For the second test, only change Maximum output speed to Very High.

Step 4: Connect Oscilloscope and Optional LED Load

Connect the oscilloscope probe to PA6.

Connect the oscilloscope ground clip to a GND pin on the NUCLEO-H753ZI.

For an optional realistic load, connect PA6 to an LED through a 330 ohm series resistor.

Keep the same load and oscilloscope settings for both Low and Very High speed measurements.

Measure the 10% to 90% rising-edge time and record the value.

Step 5: Generate the Code

After that save (Ctrl + S) and Generate Code → click Yes.

Click Yes.

The code is generated.

Scroll down until you find the while (1) infinite loop inside the main() function.

Add the toggle and delay lines inside the loop so it looks like this:

while (1)
{
  /* USER CODE END WHILE */

  /* USER CODE BEGIN 3 */
  HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_6);
  HAL_Delay(1);
}

Build the project using the Hammer (Build) icon and make sure the console reports 0 Errors, 0 Warnings.

Step 6: Flash and Test the Low-Speed Configuration

Connect your Nucleo-H753ZI board to your laptop using the USB cable.

Click the Run/Debug (Green Play / Bug icon) button to flash the code onto the board.

Observe the PA6 waveform on the oscilloscope.

Measure the 10%-90% rise time and note it in the observation table.

Also observe the fall time if required.

Step 7: Change PA6 Speed to Very High

Go back to System Core → GPIO.

Select PA6 from the GPIO pin list.

Change Maximum output speed from Low to Very High.

Keep GPIO mode as Output Push Pull.

Keep Pull-up/Pull-down as No pull-up and no pull-down.

Save the project and Generate Code again.

Build the project and flash it to the NUCLEO-H753ZI.

Capture the PA6 waveform with the same oscilloscope settings.

Measure the 10%-90% rise time and compare it with the Low-speed result.

Step 8: Configure PB0 as Floating Input

Now configure PB0 for the input test.

In the graphical Pinout View, look for pin PB0.

Click on PB0 and select GPIO_Input.

Go to System Core → GPIO and select PB0.

Set GPIO mode: Input.

Set GPIO Pull-up/Pull-down: No pull-up and no pull-down.

Leave PB0 unconnected.

Generate the code and flash the program.

Use HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_0) and observe the value in the debugger.

With no pull resistor, PB0 may show unstable or noise-sensitive readings because the input is floating.

Step 9: Configure PB0 with Pull-Up

Go to System Core → GPIO and select PB0.

Change GPIO Pull-up/Pull-down from No pull to Pull-up.

Leave PB0 unconnected.

Save, Generate Code, Build and Flash the program.

Read PB0 using HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_0).

The input should normally read HIGH.

Step 10: Configure PB0 with Pull-Down

Go to System Core → GPIO and select PB0.

Change GPIO Pull-up/Pull-down from Pull-up to Pull-down.

Leave PB0 unconnected.

Save, Generate Code, Build and Flash the program.

Read PB0 using HAL_GPIO_ReadPin(GPIOB, GPIO_PIN_0).

The input should normally read LOW.

Step 11: Program for PA6 Speed Test and PB0 Reading

Use the following GPIO configuration inside MX_GPIO_Init(). Change the PA6 speed for each test.

static void MX_GPIO_Init(void)
{
  GPIO_InitTypeDef GPIO_InitStruct = {0};

  __HAL_RCC_GPIOA_CLK_ENABLE();
  __HAL_RCC_GPIOB_CLK_ENABLE();

  HAL_GPIO_WritePin(GPIOA, GPIO_PIN_6, GPIO_PIN_RESET);

  GPIO_InitStruct.Pin = GPIO_PIN_6;
  GPIO_InitStruct.Mode = GPIO_MODE_OUTPUT_PP;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_LOW;
  HAL_GPIO_Init(GPIOA, &GPIO_InitStruct);

  GPIO_InitStruct.Pin = GPIO_PIN_0;
  GPIO_InitStruct.Mode = GPIO_MODE_INPUT;
  GPIO_InitStruct.Pull = GPIO_NOPULL;
  HAL_GPIO_Init(GPIOB, &GPIO_InitStruct);
}

For the Very High speed test, change:

GPIO_InitStruct.Speed = GPIO_SPEED_FREQ_VERY_HIGH;

For the PB0 pull-up test, change:

GPIO_InitStruct.Pull = GPIO_PULLUP;

For the PB0 pull-down test, change:

GPIO_InitStruct.Pull = GPIO_PULLDOWN;

Step 12: Observation

| PA6 Speed | Rise Time (10%-90%) | Fall Time | Observation |
| --- | --- | --- | --- |
| Low | ________ ns | ________ ns | Slower transition |
| Medium | ________ ns | ________ ns | Intermediate transition |
| High | ________ ns | ________ ns | Faster transition |
| Very High | ________ ns | ________ ns | Fastest transition |

| PB0 Configuration | Expected Reading | Observed Reading | Observation |
| --- | --- | --- | --- |
| GPIO_NOPULL | Unstable / undefined | ________ | Floating input |
| GPIO_PULLUP | HIGH | ________ | Stable HIGH |
| GPIO_PULLDOWN | LOW | ________ | Stable LOW |

Viva-voce (In-Lab):

1. Why does a faster GPIO speed setting increase electromagnetic interference?

A faster GPIO setting creates steeper signal edges. These fast transitions contain more high-frequency components and can increase EMI and ringing.

2. What is the typical resistance range of the STM32 internal weak pull resistors?

The typical range given for this experiment is approximately 30–50 kOhm.

3. In what practical scenario would you deliberately choose GPIO_SPEED_FREQ_LOW over Very High for an output pin?

For a slow control signal such as an LED or status output, where very fast switching is not required. Low speed can reduce EMI and unnecessary switching current.

4. Why is an unconnected floating digital input dangerous to leave unconfigured in a real design?

A floating input has no fixed logic level and can pick up electrical noise, causing random HIGH/LOW transitions.

Post Lab Requirements:

1. Save the screenshots/pictures of the oscilloscope output obtained and bring the same to the next lab.

2. Note the measured rise time and fall time for the different GPIO speed settings.

3. Note the PB0 readings for GPIO_NOPULL, GPIO_PULLUP and GPIO_PULLDOWN.

4. Note the procedures and observations as instructed by your advisor.

5. Submit the practical report in the next lab for correction.

Results

Rise time is expected to decrease as the PA6 GPIO speed setting increases, although the exact measured value depends on the board, probe, wiring and external load. With GPIO_NOPULL and nothing connected, PB0 can show unstable or noise-sensitive logic readings. Enabling GPIO_PULLUP gives a stable HIGH state, while GPIO_PULLDOWN gives a stable LOW state.